Setup:

- Ensure vehicle is turned off and parked safely

- Fully open the hood and ensure it is secured

- Locate the ECU in the firewall on the passenger’s side of the vehicle (See Green Arrow)

- Disconnect all 3 of the ECU connectors. Be sure to release the locking tab (Green Arrow) before moving gray connector arm.

- Connect the ECU connector provided with the X4 to the ECU and to the SCT box

- Connect the X4 to the SCT box using the OBDII cable

- Connect the SCT box to the battery using the battery clamps provided

Tune Install:

- Be sure to have the device setup using the steps above

- On the X4 select Program Vehicle

- Review and accept the Street Use Notice

- If this is your first flash, select “Upload Stock”

- Once started you will see “Saving Stock Data” run twice. This is normal.

- Once you have uploaded stock select “Download Flash”

- X4 will now program in the tune file. When complete reconnect the PCM.

Return to stock:

- Be sure to have the device setup using the steps above

- On the X4 select “Program Vehicle”

- Review and accept the Street Use Notice

- Select “Return to Stock”

- Confirm Return to Stock

- X4 will now program in the stock file

- When complete, reconnect PCM.

LiveLink Gen-II

- To use LiveLink with the 2021-2022 F150 please update to the current release version including any outstanding database updates.